VLSI Design Course

LAB 2

XOR Gate Design, Simulation, and Layout

Issue 1.2

Introduction

You will design, simulate, and lay out three transistor-level implementations of a 2-input XOR gate. You will compare implementations for functionality, propagation delay, switching energy, static/leakage current, and layout area. Finally, you will complete DRC/LVS verification.

Reading Material: Review course notes/text on:

•	Static CMOS complex gates.

•	Transmission-gate and pass-transistor logic; threshold-drop and level restoration.

•	Standard-cell layout conventions (rails, pin planning, diffusion sharing, folding).

•	DRC/LVS workflow for sky130 in Cadence.

Learning Objectives

At the end of this lab, you should be able to:

Implement three XOR topologies and explain tradeoffs (robustness, delay, energy, area).

Create a reusable testbench to verify functionality and measure delay/energy/static current.

Produce three 12-track standard-cell layouts with correct rails/pins and good routability.

Technology and Cell Framework

This lab uses the open-source SkyWater sky130 CMOS process and a fixed 12-track standard-cell architecture. All XOR cells must be layout-compatible with the IU standard-cell library used in this course.

Reference library (required): https://github.com/tdloveless/iu-vlsi/tree/main/IC_CAD/cadence/iu_stdcell_2025

Required constraints:

PDK: SkyWater sky130 (Cadence PDK used in the course environment).

Cell height: fixed 12-track (must exactly match the IU stdcell templates).

Power rails: continuous horizontal VDD (top) and VSS (bottom) aligned to the library grid.

Pin conventions: match metal layers/widths/access used in the IU stdcells (A, B inputs; Y output).

Well/substrate taps: use the same tap strategy as the reference cells; do not invent new tap spacing.

Verification: pass sky130 DRC and LVS; generate extracted views (PEX) and run post-layout simulations.

XOR Implementations (Choose 3)

Implement three (your choice) of the following XOR styles (or instructor-approved equivalents). All implementations must satisfy the 12‑track standard‑cell constraints and be evaluated using the same testbench and load conditions.

## XOR #1 — Static CMOS (Required)

Complementary pull-up/pull-down network. PMOS and NMOS transistors should be sized to match a unit-sized inverter (inv1 cell). You may use inverters to generate A’ and B’.

Why this cell?

Gold-standard for robustness and noise margin

Baseline for comparing delay, energy, and area

## XOR #2 — Transmission-Gate XOR

XOR implemented as a 2:1 mux using transmission gates

Requires inverters to generate complements

NMOS+PMOS pairs must be symmetrically laid out

Why this cell?

- Excellent balance of energy–delay–area

- Maps very naturally to standard-cell layout

## XOR #3 — NAND-Based XOR

- XOR constructed exclusively from NAND gates

- Typically implemented as a small NAND network

- Uses the NAND as a universal gate

- NAND gates may be instantiated from the IU stdcell NAND

Why this cell?

- Logic-level thinking + layout reuse

- NAND is usually the best-optimized cell in a library

## XOR #4 — AOI-Based XOR

- XOR implemented using AOI / OAI-style logic factoring

- Example: rewrite XOR into sum-of-products form that maps to AOI (Y=(AB+CD)’)

- You may use existing gates within our standard cell library to construct an AOI cell first, or build a custom AOI gate at the transistor level

Why this cell?

- Brings logic factoring + complex gates into play

- Very realistic for industrial cell libraries

## XOR #5 — Your choice

- You may choose an XOR strategy that is not listed above!

Layout

When laying out each XOR cell, pay particular attention to:

- Diffusion sharing vs pin accessibility

  - Over-sharing diffusion can break pin access in a fixed-track cell

- Internal node capacitance

  - XORs have multiple high-activity internal nodes; placement matters

- Transmission-gate symmetry

  - Asymmetry will show up as delay skew

- Output drive

  - The output Y pin must not degrade excessively

Students are expected to compare the three XOR layouts visually and explain how topology influenced routability and performance.

Test Bench

Implement the test bench using the method outlined in: https://github.com/tdloveless/iu-vlsi/blob/main/HW/tb_xor_starter.md

Grading Checklist / What to Turn In

- ☐ All three XOR cells exist: schematic, symbol, layout, extracted (PEX)

- ☐ All three layouts are exact 12-track height and align with IU stdcell rails/grid

- ☐ Pins exactly: A B Y VDD VSS, and pin access matches library conventions

- ☐ DRC clean for each XOR layout

- ☐ LVS clean for each XOR (schematic ↔ layout match)

- ☐ Stimulus exercises all input states + transitions; waveform proof included

- ☐ Worst-case tdlh/tdhl reported (schematic) for each XOR

- ☐ Energy/toggle reported (schematic)

- ☐ Static/leakage reported for all 4 DC states (00/01/10/11)

- ☐ Cell width + area reported (width × fixed height) with brief routability discussion

- ☐ Final comparison summary + conclusions included